PLUMBLINE  ·  Daily Pre-Use Fall Protection Inspection Log

Fill in one row before putting on any fall protection equipment. Every shift. Every worker. Required by O. Reg. 213/91 and CSA Z259.

| Job number or site reference |  | Fall Arrest Plan reference number |  |
| --- | --- | --- | --- |
| Competent person on site today |  | Month and year |  |

Serial numbers, anchor details, fall distance numbers, and the rescue plan are in the Site Fall Arrest Plan listed above. Use the inspection checklist on page 2 to check each item. Record pass or fail in the columns below.

| Date | Site / Location | Worker Name | Equipment ID | Harness | Lanyard | Anchor | Hardware | Overall | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

✓ = Pass    ✗ = Fail — remove equipment from service, tag it, tell your supervisor before starting work    — = Not applicable

PLUMBLINE  ·  Pre-Use Inspection Checklist — Keep This on Site

Post this page on site or keep it with the equipment. Workers complete the daily log on page 1 using these inspection points.

| What to check | Standard | Pass / Fail / N/A | Deficiency notes |
| --- | --- | --- | --- |
| FULL BODY HARNESS (CSA Z259.10) |  |  |  |
| No cuts, fraying, burns, or chemical damage to any strap | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| No broken or bent buckles, D-rings, or metal parts | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| All labels are still there and can be read | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| Stitching is intact — no loose or broken threads | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| Straps are not permanently stretched or deformed | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| Suspension trauma straps are present and undamaged (if used on this site) | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| The scheduled inspection date is current — not overdue | CSA Z259.10 | ☐ Pass ☐ Fail ☐ N/A |  |
| LANYARD / ENERGY ABSORBER / SRL (CSA Z259.11, Z259.2.5) |  |  |  |
| No cuts, fraying, or worn spots on the rope or strap | CSA Z259.11 | ☐ Pass ☐ Fail ☐ N/A |  |
| The shock absorber pack is sealed — it has not deployed or been opened | CSA Z259.11 | ☐ Pass ☐ Fail ☐ N/A |  |
| Hooks and connectors are not damaged. Gate opens, closes, and locks properly. | CSA Z259.12 | ☐ Pass ☐ Fail ☐ N/A |  |
| SRL: cable or strap is not kinked. It retracts smoothly. | CSA Z259.2.5 | ☐ Pass ☐ Fail ☐ N/A |  |
| Lanyard is the right length for the anchor and fall distance on this site | O. Reg. 213/91, s.26 | ☐ Pass ☐ Fail ☐ N/A |  |
| The scheduled inspection date is current | CSA Z259.11 | ☐ Pass ☐ Fail ☐ N/A |  |
| ANCHOR POINT (O. Reg. 213/91, s.26(4) / CSA Z259.15) |  |  |  |
| Anchor is solid — no movement, no visible damage | O. Reg. 213/91, s.26(4) | ☐ Pass ☐ Fail ☐ N/A |  |
| Load rating is at least 22 kN (5,000 lbs). Label or stamp is present and readable. | O. Reg. 213/91, s.26(4) | ☐ Pass ☐ Fail ☐ N/A |  |
| No sharp edges at the connection point that could cut the lanyard | O. Reg. 213/91, s.26(4) | ☐ Pass ☐ Fail ☐ N/A |  |
| Anchor is above the work area — swing fall risk is accounted for | O. Reg. 213/91, s.26 | ☐ Pass ☐ Fail ☐ N/A |  |
| CONNECTORS AND OTHER HARDWARE (CSA Z259.12) |  |  |  |
| Carabiners: gate opens, closes, and locks. No bending or cracking. | CSA Z259.12 | ☐ Pass ☐ Fail ☐ N/A |  |
| Cross-arm straps and beam clamps: no damage, installed correctly | CSA Z259.12 | ☐ Pass ☐ Fail ☐ N/A |  |
| Rope grabs and positioning straps: undamaged, installed correctly | CSA Z259.12 | ☐ Pass ☐ Fail ☐ N/A |  |
| AFTER ANY FALL — Remove equipment from service immediately. Do not use again until cleared by a competent person. |  |  |  |
| Has this equipment caught a fall since the last inspection? | O. Reg. 213/91 | ☐ Yes — take out of service ☐ No |  |
| If yes — tag the equipment out of service right now. Do not use it until a competent person inspects it or it is returned to the manufacturer or their authorized agent. Write up the incident. |  |  |  |

Any Fail = equipment stays on the ground. Tell your supervisor before work at height starts. Do not use damaged equipment.

Keep this log on site. Ontario law requires records to be available for inspection and kept for at least one year after the project ends. (O. Reg. 213/91, s.19)